Literature

- Key Topics to Study:

  - Monocular depth estimation (MDE) fundamentals: relative vs. metric depth.

  - Pothole detection (YOLO series, segmentation) + depth fusion.

  - Scaling methods: camera height + road plane estimation, homography, or temporal consistency from video.

  - How multiple frame from same camera be used to estimate depth

  - Challenges: scale ambiguity, lighting/shadows, domain shift (general models to roads).

- Must-Read Papers (search arXiv/Google Scholar):

  - "Robust Video-Based Pothole Detection and Area Estimation" (uses ACSH-YOLOv8 + Depth Anything V2 + MBTP + Kalman).

  - Works on monocular pothole depth with 3D scanner GT for supervision.

  - Depth Anything V2, Metric3D, Depth Pro, ZoeDepth papers.

  - Methods like StableCamH or MonoPP for metric scale in road scenes using camera height priors.

- Resources: GitHub "Awesome-Monocular-Depth", Hugging Face MDE blogs, Ultralytics docs.

Core Pipeline

- Pothole Detection — Fine-tune YOLOv8/v10/v12 or Mask R-CNN on your annotated data (Ultralytics makes this easy).

- Monocular Depth — Start with Depth Anything V2 (strong zero-shot, fine details) or MiDaS/ZoeDepth. Load via Hugging Face/PyTorch.

- Fusion & Metric Scaling:

  - Mask depth map with pothole segmentation.

  - Estimate road plane (RANSAC on assumed flat areas or semantic segmentation of road).

  - Use known camera height + pinhole model to scale relative depth to metric.

  - For video: Track potholes (BoT-SORT) + Kalman filter for stable measurements.

- Output: Pothole location, depth (max/average), area/volume approximation, severity score.

Tools: Python, PyTorch, OpenCV, Ultralytics, Hugging Face.